25-Apr-2005

Case7

24V in box(no jumper), sn003, no ferrite white cable, no ferrite between PSs, pulse connected

BUN84_25APR2005_0838_DSP2.DAT;1 and BUN90_25APR2005_0839_DSP2.DAT;1

|  | Bun84 | Bun84 | Bun90 | Bun90 |
| --- | --- | --- | --- | --- |
|  | 25apr2005_0838 | 25apr2005_0849 | 25apr2005_0839 | 25apr2005_0850 |
|  | Case7 | Case8 | Case7 | Case8 |
| Averages: | 35447.0 | 48686.6 | 49912.9 | 48748.3 |
| RMS: | 491.0 | 123.8 | 259.5 | 133.4 |

Case8

24V in box(no jumper), sn003, no ferrite white cable, no ferrite between PSs, no pulse connected (but ground of pulse carrying cable IS connected)

BUN84_25APR2005_0849_DSP2.DAT;1 and BUN90_25APR2005_0850_DSP2.DAT;1

Case9

28V multiple outlet (with jumper), sn003, no ferrite white cable, no ferrite between PSs, no pulse connected (but ground of pulse carrying cable IS connected)

BUN84_25APR2005_0920_DSP2.DAT;1 and BUN90_25APR2005_0921_DSP2.DAT;1

|  | Bun84 | Bun84 | Bun90 | Bun90 |
| --- | --- | --- | --- | --- |
|  | 25apr2005_0920 | 25apr2005_0911 | 25apr2005_0921 | 25apr2005_0912 |
|  | Case9 | Case10 | Case9 | Case10 |
| Averages: | 48726.2 | 48722.4 | 48734.1 | 48773.1 |
| RMS: | 196.3 | 113.5 | 205.4 | 120.6 |

Case10

28V multiple outlet (with jumper), sn003, no ferrite white cable, ferrite between PSs, no pulse connected (but ground of pulse carrying cable IS connected)

BUN84_25APR2005_0911_DSP2.DAT;1 and BUN90_25APR2005_0912_DSP2.DAT;1

Case11

28V multiple outlet (with jumper), sn003, no ferrite white cable, ferrite between PSs, pulse connected

BUN84_25APR2005_0937_DSP2.DAT;1 and BUN90_25APR2005_0938_DSP2.DAT;1

|  | Bun84 | Bun84 | Bun90 | Bun90 |
| --- | --- | --- | --- | --- |
|  | 25apr2005_0937 | 22apr2005_0952 | 25apr2005_0938 | 22apr2005_0953 |
|  | Case11 | Case12 | Case11 | Case12 |
| Averages: | 10456.0 | 41828.0 | 19201.5 | 49868.9 |
| RMS: | 1063.0 | 438.7 | 988.5 | 237.4 |

Case12

28V multiple outlet (WITHOUT jumper), sn003, no ferrite white cable, ferrite between PSs, pulse connected

BUN84_25APR2005_0952_DSP2.DAT;1 and BUN90_25APR2005_0953_DSP2.DAT;1