ILC Damping Rings Component/Subsystem Specification

Part I – Classification Information

| Component Description: | Quadrupole magnet power cable |  |  |  |
| --- | --- | --- | --- | --- |
| Component Location (beamline): | EDR, PDR I, PDR II | Lattice Version: |  | OCS6, 2006-06-30 |
| Document Number: | EDR-MAG-cbl-002 |  | Date: | 2006-08-03 |
| Prepared by: | Andy Wolski (Cockcroft) |  | email: | a.wolski@dl.ac.uk |
| Technical/Global System: | Magnets |  |  |  |
| Technical/Global System Contact: | John Tompkins (FNAL) |  | email: | jct@fnal.gov |
| DR Area System Contact: | Mike Zisman (LBNL) |  | email: | mszisman@lbl.gov |

Part II – Main Parameters

| Parameter | Value | # Reference |
| --- | --- | --- |
| Quantity per beamline | 614 km | [1] |
| Conductor | Copper |  |
| Electrical conductivity | 6010-6 -1m-1 |  |
| Density | 8920 kg/m3 |  |
| Conductor cost | ## Public version: no costs shown | [2] |
| Cable resistance | 0.3315 /km |  |
| Cable diameter | 8.0 mm |  |
| Cable mass | 0.4 kg/m |  |
| Maximum current | 175 A | [3] |
| Voltage drop at maximum current | 58.0 V/km |  |
| Unit cost per metre | Public version: no costs shown |  |

ILC Damping Rings Component Specification

| Document Number: | EDR-MAG-cbl-002 | Date: | 2006-08-03 |
| --- | --- | --- | --- |

Part III – Other Parameters, Information, and Drawings

Figure 1: Schematic Layout

Part IV – References

[1]	Each quadrupole separately powered.  Cable lengths based on distance to nearest alcove.

[2]	London Metal Exchange: http://www.lme.co.uk

[3]	EDR-MAG-quad-005