ILC Damping Rings Component Specification Sheet

Part I – General Information

| Component Description: | Damping Ring 6 m Bend Magnet (rev. 3) |  |  |  |
| --- | --- | --- | --- | --- |
| Component Location (beamline): | EDR, PDR1, PDR2 | Lattice Version: |  | OCS6, 2006-06-30 |
| Document Number: | EDR-MAG-dip-001 |  | Date: | 2006-08-03 |
| Prepared by: | Mark Palmer (Cornell) |  | email: | map36@cornell.edu |
| Technical/Global System: | Magnets |  |  |  |
| Technical/Global System Contact: | John Tompkins (FNAL) |  | email: | jct@fnal.gov |
| DR Area System Contact: | Mike Zisman (LBNL) |  | email: | mszisman@lbl.gov |

Part II – Main Parameters

| Parameter | Value | # Reference |
| --- | --- | --- |
| Quantity per beamline | 114 | [1] |
| Name in MAD deck | BA2F + BA2B | [2] |
| Magnet Type Designation | D50L6000 |  |
| Element Type | Sector Bend | [1] |
| Magnet technology | Normal conducting |  |
| Integrated strength k0L (nom/max) | 0.0524 / 0.0576 | [1,3] |
| Main harmonic fractional tolerance | Not yet specified |  |
| Effective length | 6.0 m | [1] |
| Slot length | 6.5 m | [4] |
| Nominal Sagitta | 0.039 m |  |
| Pole-tip gap (full) | 0.064 m | [1,7] |
| Width | 332 mm | [7] |
| Height | 140 mm | [7] |
| Core weight | 1100 kg | [7] |
| Copper weight | 265 kg | [7] |
| Total weight | 1550 kg | [7] |
| Reference radius | 0.030 m | [5] |
| Pole-tip field (nominal/maximum) | 0.146 T / 0.160 T | [1] |
| Conductor size | 13 mm × 13 mm ( 5 mm) | [7] |
| Turns per pole | 8 | [7] |
| Coil resistance | 0.029  | [7] |
| Current (nominal/maximum) | 475 A / 520 A | [7] |
| Voltage drop (nominal/maximum) | 13.7 V / 15 V | [7] |
| Power (nominal/maximum) | 6.5 kW / 7.9 kW | [7] |
| Water circuits | 4 | [7] |
| Water temperature rise at 1 MPa | 12 C | [7] |
| Unit Cost | Public version: no costs shown | [8] |

ILC Damping Rings Component Specification Sheet

| Document Number: | EDR-MAG-dip-001 (rev. 3) | Date: | 2006-08-03 |
| --- | --- | --- | --- |

Part III – Other Parameters, Information, and Drawings

| Nominal center position: x y s | 0 m 0 m various |  |
| --- | --- | --- |
| Alignment tolerance (install/BBA): x y s | Not yet specified |  |
| Angular tolerance (install/BBA): Pitch Yaw Roll | Not yet specified |  |
| Operating Characteristics Field type Polarity type Control mode | Constant field Unipolar Series |  |

Costing Information

| Steel | Public version: no costs shown |
| --- | --- |
| Conductor | Public version: no costs shown |
| Insulation | Public version: no costs shown |
| Miscellaneous | Public version: no costs shown |
| Coil Assembly | Public version: no costs shown |
| Stack Cores | Public version: no costs shown |
| Magnet Assembly | Public version: no costs shown |
| Total per Dipole | Public version: no costs shown |

Additional (1-off) costs: Tooling and construction.

Field quality specifications [6]

| N | systematic field error |  | random field error |  |
| --- | --- | --- | --- | --- |
|  | bn | an | bn | an |
| 3 | 1.6010-4 | 0 | 8.0010-5 | 0 |
| 4 | -1.6010-5 | 0 | 8.0010-6 | 0 |
| 5 | 7.6010-5 | 0 | 3.8010-5 | 0 |

It should be noted that these field quality specifications, which were those used to validate the OCS design, specify errors at the reference radius for the PEP-II dipoles.  The specifications need to be re-evaluated at a suitable reference radius for an ILC dipole design.

ILC Damping Rings Component Specification Sheet

| Document Number: | EDR-MAG-dip-001 (rev. 3) | Date: | 2006-08-03 |
| --- | --- | --- | --- |

Special notes about the damping ring bends:

- There are no changes to the magnet specifications for 500 GeV versus 1 TeV collider operation.

- At present it has not been determined whether the main bends will have trim coils.

- It is suggested that these magnets should be C-style units with coils out of the plane of the vacuum chamber to minimize synchrotron radiation issues.  This will also leave space for vacuum system antechambers as needed.

- One option for the 6 m dipole design which would minimize sagitta issues would be to build the magnet with the iron forming a chevron from two 3 m units (with full-length coils, of course).

Part IV – References

[1]	MAD deck (OCS v6), 30 June 2006.

[2] This combines 2 adjacent bend halves into a single unit for design and costing.

[3] Assumes a possible 10% increase in the ring operating energy.

[4] Slot lengths have been defined by taking the shorter of:

1) Half of the distance to the nearest neighboring magnet

2) 0.25 m

and adding twice that value to the effective length of the magnet.

[5] Uses PEP-II dipole value which was used in the lattice validation studies.  This value must be redefined for the ILC DR dipole design.

[6]	The multipole component is defined by:

ΔBy+iΔBxBr=nbn+ianxr+iyrn−1

A. Wolski, J. Gao, S. Guiducci, “Configuration Studies for the ILC Damping Rings,” LBNL-59449, pp. 21-22 (February 2006).

[7]	Magnet design by Eduard Bondarchuk, Efremov Institute, St. Petersburg, Russia.