ILC Damping Rings Component/Subsystem Specification

Part I – Classification Information

| Component Description: | Quadrupole Power Supply (1-5 kW) |  |  |  |
| --- | --- | --- | --- | --- |
| Component Location (beamline): | EDR/PDR I/PDR II | Lattice Version: |  | OCS6, 2006-06-20 |
| Document Number: | EDR-MAG-mspy-005 |  | Date: | 2006-08-03 |
| Prepared by: | Andy Wolski |  | email: | a.wolski@dl.ac.uk |
| Technical/Global System: | Magnets |  |  |  |
| Technical/Global System Contact: | John Tompkins |  | email: | jct@fnal.gov |
| DR Area System Contact: | Jie Gao Mike Zisman |  | email: | gaoj@ihep.ac.cn mszisman@lbl.gov |

Part II – Main Parameters

| Parameter | Value | # Reference |
| --- | --- | --- |
| Quantity per beamline | 341 |  |
| Maximum current | 120 A |  |
| Maximum power | 5 kW |  |
| Unipolar/bipolar | Unipolar |  |
| Unit Cost | Public version: no costs shown | [1] |

Part III – Other Parameters, Information, and Drawings

ILC Damping Rings Component Specification

| Document Number: | EDR-MAG-mspy-003 | Date: | 2006-08-03 |
| --- | --- | --- | --- |

Part IV – References

[1]	J. Chen and J. Gao, “Magnet Power System Data for Damping Rings”, 4 July 2006.