ILC Damping Rings Component Specification Sheet

Part I – General Information

| Component Description: | Electron damping ring sextupole |  |  |
| --- | --- | --- | --- |
| Component Location (beamline): | EDR, PDR1, PDR2 |  |  |
| Document Number: | EDR-MAG-sxt-001 (rev. 4) | Date: | 2006-08-17 |
| Prepared by: | Mark Palmer (Cornell) | email: | map36@cornell.edu |
| Technical/Global System: | Magnets |  |  |
| Technical/Global System Contact: | John Tompkins (FNAL) | email: | jct@fnal.gov |
| DR Area System Contact: | Jie Gao (IHEP) | email: | gaoj@ihep.ac.cn |

Part II – Main Parameters

| Parameter | Value | # Reference |
| --- | --- | --- |
| Quantity per beamline | 504 | [1] |
| Magnet Type Designation | SX30L250 |  |
| Name in MAD deck | SF, SD; SF1, SD1 | [1] |
| Max. integrated strength ¦k2L¦ | 0.252 m-2 | [1] |
| Effective length | 0.25 m | [1] |
| Pole-tip radius | 0.03 m | [1] |
| Max. pole-tip field | 0.008 T | [1] |
| Coil resistance |  |  |
| Current at nominal strength |  |  |
| Power |  |  |
| Unit Cost |  |  |

ILC Damping Rings Component Specification Sheet

| Document Number: | EDR-MAG-sxt-001 (rev. 4) | Date: | 2006-08-17 |
| --- | --- | --- | --- |

Part III – Other Parameters, Information, and Drawings

Field quality specifications at a reference radius of 0.032 m[2]. It should be noted that these field quality specifications, which were those used to validate the OCS design, specify errors at the reference radius for the PEP-II sextupoles.  The specifications need to be re-evaluated at a suitable reference radius for an ILC sextrupole design.

| N | systematic field error |  | random field error |  |
| --- | --- | --- | --- | --- |
|  | bn | an | bn | an |
| 4 | 2.010-4 |  | 1.010-4 |  |
| 5 | 1.010-4 |  | 3.010-5 |  |
| 6 | 7.010-4 |  | 1.010-4 |  |
| 7 | 1.010-4 |  | 3.010-5 |  |
| 8 | 1.010-4 |  | 3.010-5 |  |
| 9 | 1.010-4 |  | 3.010-5 |  |
| 10 | 1.010-4 |  | 3.010-5 |  |
| 11 | 1.010-4 |  | 3.010-5 |  |
| 12 | 3.210-3 |  | 1.010-4 |  |
| 13 | 1.010-4 |  | 3.010-5 |  |
| 14 | 1.010-4 |  | 3.010-5 |  |

Part IV – References

[1]	MAD deck (OCS v6, 20 June 2006).

[2]	The multipole component is defined by:

ΔBy+iΔBxBr=nbn+ianxr+iyrn−1

A. Wolski, J. Gao, S. Guiducci, “Configuration Studies for the ILC Damping Rings,” LBNL-59449, pp. 21-22 (February 2006).