- ILC Damping Rings Component Specification Sheet

- Part I – Classification Information

| Component Description: | Generic Damping Ring Sextupole |  |  |  |
| --- | --- | --- | --- | --- |
| Component Location (beamline): | EDR/PDR I/PDR II | Lattice Version: |  | OCS6, 2006-06-20 |
| Document Number: | EDR-MAG-sxt-002 |  | Date: | 2006-07-12 |
| Prepared by: | Jin-Young Jung |  | email: | JYJung@lbl.gov |
| Technical/Global System: | Magnets |  |  |  |
| Technical/Global System Contact: | John Tompkins |  | email: | jct@fnal.gov |
| DR Area System Contact: | Mike Zisman |  | email: | mszisman@lbl.gov |

- Part II – Main Parameters

| Parameter | Value | # Reference |
| --- | --- | --- |
| Quantity per beamline | 480 | [1] |
| Length | 250 m |  |
| Pole-tip radius | 30 mm |  |
| Maximum pole-tip field in lattice | 0.00720 T |  |
| Minimum pole-tip field in lattice | 0.00467 T |  |
| Maximum design pole-tip field | 0.00756 T | [2] |
| Total current per pole | 64.4 Ampere-turns |  |
| Current density | 95.3 Amperes/cm2 |  |
| Conductor | #10 AWG square conductor, heavy insulation |  |
| Number of turns | 19 |  |
| Current | 7.2 Amperes |  |
| Voltage | 0.53 Volts |  |
| Power per magnet | 3.8 W |  |
| Cooling water power rise | No water cooling |  |
| Cooling water flow rate | No water cooling |  |
| Field quality (dg/g) | 0.54% at r = 2.5 cm |  |
| Magnet efficiency | 98.3% |  |
| Core volume | 0.004 m3 |  |
| Core mass | 73.12 lb |  |
| Coil mass | 4.05 lb |  |
| Core cost | ## Public version: no cost information shown | [3] |
| Coil cost | Public version: no cost information shown | [3] |
| Assembly and test | Public version: no cost information shown | [3] |
| Unit Cost | Public version: no cost information shown |  |

- ILC Damping Rings Component Specification Sheet

| Document Number: | EDR-MAG-sxt-002 | Date: | 2006-07-12 |
| --- | --- | --- | --- |

- Part III – Other Parameters, Information, and Drawings

Field in poles ~ 0.0107 T Field in yoke ~ 0.0305 T

- ILC Damping Rings Component Specification Sheet

| Document Number: | EDR-MAG-sxt-002 | Date: | 2006-07-12 |
| --- | --- | --- | --- |

Part IV – References

[1]	Includes only sextupoles with non-zero values in OCS6.

[2]	Actual maximum design pole-tip field is 0.00794 T; the quoted value is the effective maximum design pole-tip field, allowing 5% reduction from end effects.

[3]	Scaled from ALS costs, plus inflation increase of 4% per year from 1989 to 2006.