ILC Damping Rings Component/Subsystem Specification

Part I – Classification Information

| Component Description: | Electron damping ring klystron |  |  |  |
| --- | --- | --- | --- | --- |
| Component Location (beamline): | EDR | Lattice Version: |  | OCS2, March 2006 |
| Document Number: | EDR-RF-kly-001 |  | Date: | 2006-06-12 |
| Prepared by: | Roberto Boni |  | email: | roberto.boni@lnf.infn.it |
| Technical/Global System: | RF Power |  |  |  |
| Technical/Global System Contact: | Shigeki Fukuda |  | email: | shigeki.fukuda@kek.jp |
| DR Area System Contact: | Susanna Guiducci |  | email: | susanna.guiducci@lnf.infn.it |

Part II – Main Parameters

| Parameter | # Value |  | # Reference |
| --- | --- | --- | --- |
| Quantity per beamline | 8 | (7) | [1] |
| RF Frequency | 650 MHz |  |  |
| Max output power | 800 kW |  |  |
| Actual output power | 436 kW | (500 kW) | [1] |
| Efficiency | 62 % |  |  |
| Gain | 40 dB |  |  |
| Input power | 44 W | (50 W) | [1] |
| HV | 50 - 75 kV |  |  |
| Beam current | 10 - 17 A |  |  |
| Electromagnet DC power | 1500 W |  |  |
| Collector coolant flow | 1400 l/min |  |  |
| Dimensions (H,L,W) | 4, 1.6, 1.5 m |  |  |
| Weight | 2200 kg |  |  |
| Unit Cost | Public version: no cost shown |  |  |

Part III – Other Parameters, Information, and Drawings

ILC Damping Rings Component Specification

| Document Number: | EDR-RF-kly-001 | Date: | 2006-06-12 |
| --- | --- | --- | --- |

Part IV – References

[1]	Total cost should be based on 8 RF stations (klystrons) in EDR.  Performance specification based on being able to operate with 7 RF stations running.

[2]	R. Boni, G. Cavallari, RF System for the ILC Damping Rings, ILC-LNF-001, Tech. Note, June 2006.