ILC Damping Rings Component/Subsystem Specification

Part I – Classification Information

| Component Description: | Electron Damping Ring RF Waveguide System |  |  |  |
| --- | --- | --- | --- | --- |
| Component Location (beamline): | EDR | Lattice Version: |  | OCS2, March 2006 |
| Document Number: | EDR-RF-wvg-001 |  | Date: | 2006-06-12 |
| Prepared by: | Roberto Boni |  | email: | roberto.boni@lnf.infn.it |
| Technical/Global System: | RF Systems |  |  |  |
| Technical/Global System Contact: |  |  | email: |  |
| DR Area System Contact: | Susanna Guiducci |  | email: | susanna.guiducci@lnf.infn.it |

Part II – Main Parameters

| Parameter | # Value | # Reference |
| --- | --- | --- |
| Waveguide type/material/designation | Rectangular/Aluminium/WR1500 |  |
| Waveguide inside dimensions | 15” x 7.5” |  |
| Waveguide weight | ≈ 5 kg · meter |  |
| Waveguide total length per RF station | ≈ 200 m |  |
| Attenuation @ nominal frequency | 0.165 dB/100m |  |
| Waveguide power loss per RF station | ≈ 1 % |  |
| Quantity of Magic Tees | 24 |  |
| Quantity of 3-port Circulators | 8 |  |
| Cost per ring | Public version: no cost shown |  |

Part III – Other Parameters, Information, and Drawings

ILC Damping Rings Component Specification

| Document Number: | EDR-RF-wvg-001 | Date: | 2006-06-12 |
| --- | --- | --- | --- |

Part IV – References

[1] R. Boni, G. Cavallari, RF System for the ILC Damping Rings, ILC-LNF-001, Tech. Note, June 2006.