ILC Damping Rings Component/Subsystem Specification

Part I – Classification Information

| Component Description: | Electron DR Wiggler Straight Quadrupole Vacuum Chamber |  |  |  |
| --- | --- | --- | --- | --- |
| Component Location (beamline): | EDR | Lattice Version: |  | OCS6, 2006-06-30 |
| Document Number: | EDR-VAC-chmb-004 |  | Date: | 2006-08-22 |
| Prepared by: | Steve Marks (LBNL) Dave Plate (LBNL) |  | email: | s_marks@lbl.gov dwplate@lbl.gov |
| Technical/Global System: | Vacuum |  |  |  |
| Technical/Global System Contact: | John Noonan |  | email: | noonan@aps.gov |
| DR Area System Contact: | Andy Wolski |  | email: | a.Wolski@dl.ac.uk |

Part II – Main Parameters

| Parameter | Value | # Reference |
| --- | --- | --- |
| Quantity per beamline | 80 |  |
| Operating pressure | 110-9 torr | [1] |
| Length | 1.0 meter |  |
| Unit Cost | Public version: no costs information shown |  |

Part III – Other Parameters, Information, and Drawings

Keep power density below 15 W/mm2 Keep physical cross sectional size to fit within the quadrupole design (30 mm magnetic aperture). Length= 1.0 m Chamber designed without absorbers. All power absorbed within wiggler chambers. Surface coating for reduction of secondary electrons. (Will be required for the PDR’s, assume coating of all chambers) Currently does not include clearing electrodes. Estimate includes a 40 l/s ion pump with power supply, a bellows assembly with RF interface, and a BPM assembly.

ILC Damping Rings Component Specification

| Document Number: | EDR-VAC-chmb-004 | Date: | 2006-08-22 |
| --- | --- | --- | --- |

Part III – Other Parameters, Information, and Drawings (continued)

| Item | Unit | Units/chamber | $/Unit | $/m |
| --- | --- | --- | --- | --- |
| Aluminum | Kg | 23.1 |  |  |
| 1Machining (Coarse) | hr | 1 |  |  |
| 1Machining (Fine) | hr | 1.5 |  |  |
| NEG Coating | m | 1 |  |  |
| Welding | hr | 2 |  |  |
| Cleaning | hr | 2 |  |  |
| Fixtures | m | 1 |  |  |
| Ion Pump 40 l/s | m | 1 |  |  |
| IP Power Supply | m | 1 |  |  |
| BPM/Bellows Assembly |  |  |  |  |
| Bellows/RF Assembly | m | 1 |  |  |
| BPM Buttons | ea | 4 |  |  |
| Machining/Welding | hr | 2 |  |  |
| Total |  |  |  |  |

Cross-section of wiggler section quadrupole vacuum chamber.  The chamber is NEG-coated.

ILC Damping Rings Component Specification

| Document Number: | EDR-VAC-chmb-004 | Date: | 2006-08-22 |
| --- | --- | --- | --- |

Part IV – References

[1]	A. Wolski, J. Gao, S. Guiducci, “Configuration Studies and Recommendations for the ILC Damping Rings”, LBNL-59449, February 2006.