ILC Damping Rings Component/Subsystem Specification

Part I – Classification Information

| Component Description: | Pulser for single positron damping ring injection/extraction kicker |  |  |  |
| --- | --- | --- | --- | --- |
| Component Location (beamline): | PDRI | Lattice Version: |  | OCS6, 2006-06-30 |
| Document Number: | PDRI-INX-plsr-001 |  | Date: | 2006-08-22 |
| Prepared by: | Tom Mattison (UBC) |  | email: | mattison@physics.ubc.ca |
| Technical/Global System: | Magnets |  |  |  |
| Technical/Global System Contact: | John Tompkins (FNAL) |  | email: | jct@fnal.gov |
| DR Area System Contact: | Mike Zisman (LBNL |  | email: | mszisman@lbl.gov |

Part II – Main Parameters

| ## Parameter | Value | # Reference |
| --- | --- | --- |
| Beam energy | 5 GeV | [1] |
| Acceptance, Ax,max | 0.09 m-rad | [1] |
| Minimum bunch spacing | 3 ns | [2] |
| Assumed stripline geometry factor | 0.7 | [3] |
| Total required kicker amplitude | 131 kV-m | [4] |
| Stripline length | 20 cm | [5] |
| Kicker rise time | < 1.67 ns | [5] |
| Kicker fall time | < 1.67 ns | [5] |
| Kicker flat top | 1.33 ns | [5] |
| Total kick voltage amplitude | 657 kV | [6] |
| Module voltage | 10 kV | [7] |
| Number of modules required | 66 |  |
| Pulser voltage | 5 kV | [8] |
| Number of pulsers required | 132 | [9] |
| Unit cost per pulser | Public version: no costs information shown | [8] |

Part III – Other Parameters, Information, and Drawings

ILC Damping Rings Component Specification

| Document Number: | PDRI-INX-plsr-001 | Date: | 2006-08-22 |
| --- | --- | --- | --- |

Part IV – References

[1]	Baseline Configuration.  See also: A. Wolski, J. Gao, S. Guiducci (editors) “Configuration Studies and Recommendations for the ILC Damping Rings,” LBNL-59449, February 2006.

The acceptance is defined as the maximum value of Ax for any particle in the beam, where:

| Axγ=γxx2+2αxxpx+βxpx2 | (1) |
| --- | --- |

where γ is the relativistic factor.  Note that the largest transverse coordinate of any particle in the beam at a location with zero dispersion is:

| xmax=βxAx,maxγ | (2) |
| --- | --- |

[2]	Baseline Configuration, for 4346, 5658 or 5782 bunches (bunch populations 1.29×1010, 0.99×1010, 0.97×1010 respectively).  See H. Ehrlichmann, S. Guiducci, K. Kubo, M. Kuriki, A. Wolski, “Recommendations for ILC Configuration Satisfying Timing Constraints”, April 2006.  https://wiki.lepp.cornell.edu/ilc/bin/view/Public/DampingRings/WebHome

[3]	For striplines with cross-section forming arcs of a circle and subtending an angle of φ at the center of the circle, the geometry factor is:

| k=4πsinϕ2 | (3) |
| --- | --- |

For φ = 90°, the geometry factor is k = 0.9.  A more conservative assumption, taking into account engineering constraints etc., would be k = 0.7.

[4]	The deflection angle from the kickers is:

| θ=2kVgLEe | (4) |
| --- | --- |

where k is the geometry factor, V is the voltage between the striplines, g is the (full) gap between the striplines, L is the length of the striplines, and E is the beam energy.  The horizontal offset at the septum from the deflection θ at the kickers is:

| Δx=R12θ=θβx,kβx,ssinμks | (5) |
| --- | --- |

where R12 is the matrix element from the kicker to the septum, βx,k and βx,s are the beta functions at the kicker and at the septum respectively, and μks is the phase advance from the kicker to the septum.  If we assume (making no allowance for beam stay-clear or thickness of the septum blade):

ILC Damping Rings Component Specification

| Document Number: | PDRI-INX-plsr-001 | Date: | 2006-08-22 |
| --- | --- | --- | --- |

| Δx=2βx,sAx,maxγ | (6) |
| --- | --- |

and that the minimum gap between the striplines (again making no allowance for beam stay-clear) is:

| g=2βx,kAx,maxγ | (7) |
| --- | --- |

and finally assume the optimum phase advance μks = 90°, then we find from equations (4)-(7):

| VL=2kAx,maxE/eγ | (8) |
| --- | --- |

Using k = 0.7 [3], Ax,max = 0.09 m-rad [1], and E = 5 GeV [1], we find:

| VL=131.4 kV-m | (9) |
| --- | --- |

Note that this required kicker amplitude makes no allowance for beam stay-clear at the kicker or septum, or for the thickness of the septum blade.

[5]	The stripline length adds to the width of the voltage pulse from the pulser to give a total effective pulse width.  Assuming a pulse rise time of τrise, an optimum flat-top of τflat = 2L/c (where L is the stripline length) and a fall time of τfall, the minimum bunch separation is:

| τsep=maxτrise+2Lc,2Lc+τfall | (10) |
| --- | --- |

A stripline length of 30 cm allows a maximum 1 ns rise or fall time for 3 ns bunch separation.  Decreasing the stripline length to 30 cm increases the allowed rise or fall time to 1.6 ns.

[6]	Required total kicker voltage V = (VL)/L.

[7]	Assuming two pulsers per module, one providing +5 kV with respect to ground, the other providing –5 kV with respect to ground.

[8]	FID GmbH, Carl-Benz-Strasse 28, Industriepark, 57299, Burbach, Germany.

http://www.fidtechnology.com/Products/fastpowergens.htm

[9]	Number of pulsers required per kicker.  Each ring requires one injection and one extraction kicker.  Here, we have assumed that the injection and extraction kickers are identical, and are driven by the size of the injected positron beam.  Some cost savings may be achieved by taking advantage of the smaller deflection required for the extraction kickers, because of the smaller size of the extracted beam.