ILC Damping Rings Component/Subsystem Specification

Part I – Classification Information

| Component Description: | Positron damping ring splitter system |  |  |  |
| --- | --- | --- | --- | --- |
| Component Location (beamline): | PDRI | Lattice Version: |  | OCS6, 2006-06-30 |
| Document Number: | PDRI-SPLT-sys-001 |  | Date: | 2006-08-22 |
| Prepared by: | Tom Mattison (UBC) |  | email: | mattison@physics.ubc.ca |
| Technical/Global System: | Magnets |  |  |  |
| Technical/Global System Contact: | John Tompkins (FNAL) |  | email: | jct@fnal.gov |
| DR Area System Contact: | Mike Zisman (LBNL) |  | email: | mszisman@lbl.gov |

Part II – Main Parameters

| Parameter | Value | # Reference |
| --- | --- | --- |
| Kicker amplitude | 131 kV-m | [1] |
| Kicker rise/fall time | <180 ns | [2] |
| Kicker stripline length | 2 m |  |
| Pulser output voltage | 5 kV | [3] |
| Number of pulsers per kicker unit | 2 | [4] |
| Kick amplitude per kicker unit | 20 kV-m |  |
| Number of kicker units needed | 7 |  |
| Stripline costs per kicker unit | ## Public version: no costs information shown |  |
| Cost per pulser | Public version: no costs information shown |  |
| Septum type | DC |  |
| Septum cost | Public version: no costs information shown |  |
| Septum power supply cost | Public version: no costs information shown |  |
| Number of dipoles | ## To be specified |  |
| Dipole cost | Public version: no costs information shown | [5] |
| Dipole power supply cost | Public version: no costs information shown | [6] |
| Number of quadrupoles | To be specified |  |
| Quadrupole cost | Public version: no costs information shown | [7] |
| Quadrupole power supply cost | Public version: no costs information shown | [8] |
| System Cost | Public version: no costs information shown |  |

Part III – Other Parameters, Information, and Drawings

ILC Damping Rings Component Specification

| Document Number: | PDRI-SPLT-sys-001 | Date: | 2006-08-22 |
| --- | --- | --- | --- |

Part IV – References

[1]	Component Specification Sheet: PDR1I-INX-plsr-001.

[2]	Bunch spacing is 181 ns for 5658 bunches per machine pulse (bunch population 0.99×1010 particles).  See H. Ehrlichmann, S. Guiducci, K. Kubo, M. Kuriki, A. Wolski, “Recommendations for ILC Configuration Satisfying Timing Constraints”, April 2006.  https://wiki.lepp.cornell.edu/ilc/bin/view/Public/DampingRings/WebHome

[3]	FID GmbH, Carl-Benz-Strasse 28, Industriepark, 57299, Burbach, Germany.

http://www.fidtechnology.com/Products/fastpowergens.htm

[4]	Two pulsers: one provides +5 kV on one stripline; the other provides –5 kV on the other stripline.

[5]	Component Specification Sheet: EDR-MAG-dip-002.

[6]	Component Specification Sheet: EDR-MAG-mspy-002.

[7]	Component Specification Sheet: EDR-MAG-quad-005.

[8]	Component Specification Sheet: EDR-MAG-mspy-006.