ILC Damping Rings Component Specification Sheet

Part I – General Information

| Component Description: | Positron Damping Ring 1 Bend Magnet (rev. 2) |  |  |
| --- | --- | --- | --- |
| Component Location (beamline): | PDR1 |  |  |
| Document Number: | EDR-MAG-dip-002 | Date: | 2006-06-15 |
| Prepared by: | Mark Palmer (Cornell) | email: | map36@cornell.edu |
| Technical/Global System: | Magnets |  |  |
| Technical/Global System Contact: | John Tompkins (FNAL) | email: | jct@fnal.gov |
| DR Area System Contact: | Mike Zisman | email: | mszisman@lbl.gov |

Part II – Main Parameters

| Parameter | Value | # Reference |
| --- | --- | --- |
| Quantity per beamline | 20 | [1] |
| Name in MAD deck | BA2 | [2] |
| Magnet Type Designation | D50L3000 |  |
| Element Type | Sector Bend | [1] |
| Integrated strength k0L (nom/max) | 0.00262 / 0.00288 | [1,3] |
| Main harmonic fractional tolerance | Not yet specified |  |
| Effective length | 3.0 m | [1] |
| Slot length | 3.5 m | [4] |
| Nominal Sagitta | 0.010 m |  |
| Pole-tip gap (full) | 0.050 m | [1] |
| Reference radius | 0.030 m | [5] |
| Pole-tip field (nom/max) | 0.146 T / 0.160 T | [1] |
| Nominal center position: x y s | 0 m 0 m various |  |
| Alignment tolerance (install/BBA): x y s | Not yet specified |  |
| Angular tolerance (install/BBA): Pitch Yaw Roll | Not yet specified |  |
| Magnet Technology | Normal conducting |  |
| Operating Characteristics Field type Polarity type Control mode | Constant field Unipolar Series |  |
| Coil resistance |  |  |
| Current at nom/max strength |  |  |
| Power (nom/max) |  |  |
| Unit Cost | No cost yet provided |  |

ILC Damping Rings Component Specification Sheet

| Document Number: | EDR-MAG-dip-002 | Date: | 2006-06-15 |
| --- | --- | --- | --- |

Part III – Other Parameters, Information, and Drawings

Field quality specifications [6]

| N | systematic field error |  | random field error |  |
| --- | --- | --- | --- | --- |
|  | bn | an | bn | an |
| 3 | 1.6010-4 | 0 | 8.0010-5 | 0 |
| 4 | -1.6010-5 | 0 | 8.0010-6 | 0 |
| 5 | 7.6010-5 | 0 | 3.8010-5 | 0 |

It should be noted that these field quality specifications, which were those used to validate the OCS design, specify errors at the reference radius for the PEP-II dipoles.  The specifications need to be re-evaluated at a suitable reference radius for an ILC dipole design.

Special notes about the damping ring bends:

- There are no changes to the magnet specifications for 500 GeV versus 1 TeV collider operation.

- At present it has not been determined whether the main bends will have trim coils.

- It is suggested that these magnets should be C-style units with coils out of the plane of the vacuum chamber to minimize synchrotron radiation issues.  This will also leave space for vacuum system antechambers as needed.

Part IV – References

[1]	MAD deck (OCS v2), 23 March 2006.

[2] This combines 2 adjacent bend halves into a single unit for design and costing.

[3] Assumes a possible 10% increase in the ring operating energy.

[4] Slot lengths have been defined by taking the shorter of:

1) Half of the distance to the nearest neighboring magnet

2) 0.25 m

and adding twice that value to the effective length of the magnet.

[5] Uses PEP-II dipole value which was used in the lattice validation studies.  This value must be redefined for the ILC DR dipole design.

[6]	The multipole component is defined by:

ΔBy+iΔBxBr=nbn+ianxr+iyrn−1

A. Wolski, J. Gao, S. Guiducci, “Configuration Studies for the ILC Damping Rings,” LBNL-59449, pp. 21-22 (February 2006).