Fall 1991

Physics 310

Franck-Hertz Experiment

References

Melissinos, Experiments in Modern Physics, (1966) pp. 8-17

Hoag and Korf, Electron and Nuclear Physics, 3rd Edition (1947), PP 151-162

Harnwell and Livingood, Experimental Atomic Physics (1933),pp 314-323

Franck-Hertz Tube, Instructions for Use

Arnot and Baines, Elastic and Inelastic Cross Sections of the Mercury Atom,

Proc. Roy. Soc . Lond., 151 (1936) 256.

N. Frank, Introduction to Electricity and Optics pp 221-223 about I vs V3/2

Richard S. Galik, Ramp Generator for Franck-Hertz (1990), lab handout

In 1914, one year after Bohr proposed his model of the hydrogen atom with discrete energy levels, James Franck and Gustav Hertz performed a brilliant experiment to demonstrate that mercury atoms could be raised from their ground state to a higher energy level by inelastic collisions with electrons.  The experiment gave strong support to the concept of energy levels by showing that the mercury atoms could not take up any of the electrons' energy until that energy became great enough to raise the atoms to the first level.  The energies of the levels that can be reached by electron excitation are generally called critical potentials  and are expressed in Volts (meaning electron Volts).

In this experiment you can measure the first critical potential of mercury and the ionization potential, i.e. the energy required to free an electron from the atom.  The action takes place in a simple triode which is constructed in a manner to minimize leakage currents between electrodes.  The tube is evacuated but contains a small quantity of mercury, which at room temperature has a vapor pressure of only 10-3 Torr but at elevated temperatures the vapor pressure rises rapidly (see the graph).  By changing the temperature of the tube you can change the vapor pressure and hence the density of mercury atoms in the tube.  Do not run the temperature above 190˚ C.

A simplified schematic circuit for measuring the first critical potential is shown in Fig 1.  The filament of the tube can be heated by a 6 Volt battery with a variable series resistor, or by a variac and filament transformer.  The voltage U is a small fixed valve, 1.5 Volts or less.  The voltage V is supplied by a  variable power

supply.  The meter A is a picoammeter whose output can be fed to an x-y plotter on which you can plot directly curves of I vs V.

An electron that suffers only elastic collisions has a potential energy which varies only with its position in the tube and in the absence of space charge is of the form shown in

Fig. 2.  If V > U electrons that pass the grid have no problem reaching the plate.  But when V reaches the critical potential some electrons will lose their energy by undergoing inelastic scattering near the grid and will not be able to reach the plate.  We should expect to see a sudden drop in the plate current at this value of V.  By choosing an optimum value of  the mean free path between inelastic collisions, you will find that you can see a sharp drop in plate current at the critical potential Vc

and subsequent drops at 2Vc, 3Vc, etc. where the electrons suffer 2, 3 etc. inelastic collisions on their travels from the cathode to the grid.

The mean free path between two successive collisions (λ) is related to the number of atoms per unit volume (n) and the cross section for the collisions (σ) by

λ = \f(1,nσ)

The quantity σ can be thought of as the area an atom presents to an electron for the type of collision in question.  The measured σ for the collision that raises the mercury atom from the ground state to the first excited state is plotted in Fig. 7 of Arnot and Baines.  At the threshold energy the cross section rises rapidly to about 4 x 10-16 cm2.  In order to get a sharp drop in the electron current when we reach the critical potential we need a mean free path that is much shorter than the cathode-grid separation which is about one centimeter.  You can get  a good idea of what temperature is required by plotting a graph of mean free path against temperature, using the above cross section and the ideal gas law to convert vapor pressure to density of atoms.  Having such a graph on hand when you are taking data will help you understand what is going on inside the tube.

The circuit can be modified as shown in Fig. 3 to find the ionization potential.  In this case U can be about 5 Volts and since it returns to the cathode the plate potential is always below the cathode, and electrons cannot reach the plate.  You should draw a potential energy diagram similar to Fig. 2 to see how this works.  Under the proper conditions you can expect to observe

a sudden jump in positive plate current at a value of V corresponding to the ionization potential.  Electrons accelerated toward the grid can actually pass through it and oscillate back and forth several times before being collected.  Any ions created on the plate side of the grid will be attracted to the plate and register on the meter.

In addition to measuring the first critical potential and the ionization potential of mercury, you should vary one or more of the parameters (U or the temperature) and observe the effect on your results.  You should also read about contact potential in the references and estimate the contact potential between the cathode and grid from your results.

J.DeWire, Aug. '80

revised May '92, E.C.

1 Torr = 1mm Hg = 133 Pa

Now you need a longer mean free path, i.e. lower temperature and vapor pressure so that the electrons can actually reach the grid and produce ions in that region.  At lower vapor pressure there will be less atoms to ionize; so you will have to settle for a compromise temperature.