Procedures for Handling Physics 310 Gamma Sources

(January 1990)

1.	Make entry as indicated in P310 Gamma Source Logbook (date and time, source identification, 	type of transfer and use, your name).

2.	Unlock storage box, open cover of lead container.  Pick up source source at tagged end and 	transfer it directly to its location in the experiment.

3.	Close lead storage container.

4.	Place sign:  CAUTION, RADIOACTIVE MATERIAL next to equipment.

5.	Check work area with survey meter whenever you use a source for the first time.

6.	Maintain surveillance of your work area while the source is in the experiment, i.e. you 	must be in the room while the source is out.  The source must be put away and locked if you 	need to leave for any reason.

7.	When finished with the source, open cover of lead storage container before taking the source 	out of the experiment.  Move it directly back into storage container.  Close lead cover.

8.	Remove sign.

9.	Enter return of source in P310 Gamma Source Logbook

10. Make new entry if you use another source, and proceed as described in 2.) - 9.)

11. Lock storage box.

12. Make final entry in P310 Gamma Source Logbook .  Return key to its place in 430 Clark Hall.

Note:  430 Clark Hall is locked outside of class hours.  Check with your instructor if you need access at other times.